CANOPY

Scoring — Results & Benchmark

Numerical results, plain-English definitions, accuracy, and benchmark

Luna · Geospatial Lead · Project Canopy · GCL Hackathon 2026

# 1 · Full numerical results (both demo projects)

Every value the engine produced on the latest run.

| Metric | Nur Navoi (solar) | Mikoko (mangrove) |
| --- | --- | --- |
| AOI area (ha) | 544.7 | 177.5 |
| Sentinel-2 scenes (before / after) | 22 / 24 | 13 / 5 |
| Screening status | Partially consistent | Partially consistent |
| Risk score / band | 5 / Low | 5 / Low |
| Confidence | 0.87 (high) | 0.60 (medium) |
| Change detected | true (build) | true (vegetation) |
| Human review recommended | false | true |
| NDVI start → end | n/a (solar) | 0.404 → 0.476 |
| NDVI trend (end − start) | n/a | +0.072 (endpoint — not robust; see slope) |
| NDVI annual slope /yr | n/a | −0.012 |
| Vegetation change % (gain / loss) | 0.1% | 44% (39% gain / 4% loss) — bi-temporal 2019↔2024, noise-level; no significant trend |
| Carbon estimate (tCO₂e) | n/a | 8,667.7 |
| Above-ground biomass (t/ha, ESA CCI) | n/a | 28.3 |
| Fire — Fosberg FFWI | 18.4 | 3.8 |
| Fire risk (FFWI × fuel, 0–100) | 5 | 4 |
| Fire severity — dNBR (class) | 0.003 (Unburned) | −0.111 (Enhanced regrowth) |
| Flood — observed (Otsu) | 0.0% | 0.0% |
| Flood — 100-yr depth (Aqueduct) | 0.000 m | 0.093 m |
| Flood exposure damage (Huizinga) | 0% | 4.6% |
| Degradation hazard (0–100) | 0 | 4 |
| Community exposure (GHSL built-up) | 0.00 | 0.03 |
| Build signal — SAR backscatter Δ | +1.2 dB (detected) | n/a (vegetation asset — build detection applies to infrastructure only) |
| Annual GHI (kWh/m²/yr, ERA5) | 1,869 | n/a |
| Est. generation — PVWatts (GWh/yr) | 150 (fixed) – 194 (tracking) | n/a |
| Est. avoided emissions (tCO₂/yr, PVWatts × grid factor) | 82k – 107k | n/a |

Mikoko NDVI annual trajectory (same season each year): 2019 = 0.40, 2020 = 0.63, 2021 = 0.45, 2022 = 0.38, 2023 = 0.39, 2024 = 0.48 (slope −0.012/yr — noisy, roughly flat: a mature, stable mangrove).

# 2 · Metric definitions (plain English)

| Metric | What it means |
| --- | --- |
| AOI area | Size of the analyzed boundary, in hectares. |
| Sentinel-2 scenes (before/after) | Number of usable, cloud-filtered optical images per window. More = cleaner composite (minimum 3). |
| Screening status | The verdict: Consistent / Partially / Inconsistent / Insufficient data. |
| Risk score / band | 0–100 physical-risk number + Low/Medium/High/Critical. |
| Confidence | 0–1 trust level, from scene count + cloud-free pixel coverage. |
| Change detected | Whether meaningful change occurred (vegetation or build). |
| Human review recommended | Flag: a person should check (low confidence or lots of change). |
| NDVI start → end | Mean greenness at the start vs end window. |
| NDVI trend / annual slope | Two-point change / trend fit through all yearly points (more robust). |
| Vegetation change % (gain/loss) | % of area whose greenness changed, split into growth vs loss. |
| Carbon estimate | Estimated stored carbon, tonnes CO₂-equivalent (from biomass). |
| Above-ground biomass | Biomass density (t/ha) from ESA CCI — the input to carbon. |
| Fire — Fosberg FFWI | Fire-WEATHER danger (0–100) from temperature, humidity, wind. |
| Fire risk (FFWI × fuel, 0–100) | That danger scaled by fuel present (vegetation to burn). |
| Fire severity — dNBR | Did an actual fire BURN between the dates. Negative = vegetation gained. |
| Flood — observed (Otsu) | % of area ACTUALLY newly flooded in the window. |
| Flood — 100-yr depth (Aqueduct) | Forward-looking: how deep a 1-in-100-year flood would be here. |
| Flood exposure damage (Huizinga) | That depth converted to % damage via the depth→damage curve (the flood risk sub-score). |
| Degradation hazard (0–100) | Risk sub-score from vegetation LOSS (% of area lost). |
| Community exposure (GHSL) | How much settlement/buildings are nearby (context, not merged into the score). |
| Build signal — SAR Δ | Radar-brightness change that detects construction. |
| Annual GHI | Yearly solar energy reaching the ground (kWh/m²), from ERA5. |
| Est. generation (PVWatts) | Modeled expected output (capacity × solar resource) — an ESTIMATE, not a measurement. |
| Est. avoided emissions | Modeled avoided emissions = generation estimate × grid emission factor — an ESTIMATE. |

# 3 · Accuracy

| Check | Result |
| --- | --- |
| Mikoko carbon vs reported credits | 8,668 tCO₂e vs ~9,880 — within ~12% |
| Nur Navoi build vs reported (2021) | Detected (+1.2 dB), consistent with 2021 |
| Nur Navoi generation (PVWatts estimate) | 150–194 GWh/yr vs reported 270 — our estimate is a conservative lower bound (horizontal GHI, no tilt/tracking); the gap reflects tilt/tracking, not over-reporting |
| Emissions avoided (tCO₂/yr) | Our estimate ≈ 82–107k tCO₂/yr (generation × CDM Uzbekistan grid factor) vs reported 156k — a conservative lower bound, tracking our generation estimate |
| Biodiversity | Not observable from Sentinel — proxies only |

# 4 · Benchmark vs reported figures

## Mikoko Pamoja (mangrove)

| Our observation | Benchmark | Near / far |
| --- | --- | --- |
| AOI box ~177 ha; mangrove present | 117 + 10 ha protected / restored | Same scale (box larger) |
| Carbon stock ~8,700 tCO₂e (ESA CCI) | ~9,880 tCO₂ credits (2014–18) | Same order — stock vs credits differ |
| 2019–24 NDVI roughly flat | Biomass doubled over 12 yr (to ~2018) | Our window post-dates the growth |
| 39% gain / 4% loss; NDVI flat (slope -0.012/yr, within interannual noise); low risk | No deforestation / permanence | Partially consistent — no significant loss |
| Carbon aligned to Plan Vivo | Plan Vivo certified | Method-aligned |

Sources for reported figures (confirm each): Plan Vivo — Mikoko Pamoja (Kenya) project page and the project's Plan Vivo design document / annual reports (planvivo.org/projects/mikoko-pamoja-kenya).

## Nur Navoi (solar)

| Our observation | Benchmark | Near / far |
| --- | --- | --- |
| Build detected (+1.2 dB); panels visible | 100 MW built, operational Aug 2021 | Confirmed |
| Backscatter rises 2019→2024 | Operational since 2021 | Consistent with 2021 |
| PVWatts estimate 150–194 GWh/yr (100 MW + ERA5 GHI); conservative lower bound — horizontal GHI, no tilt/tracking/bifacial | ~270 GWh/yr generation | Consistent — our PVWatts estimate is a conservative lower bound; the gap to 270 reflects our simplification (tilt / tracking / plane-of-array), not over-reporting |
| ≈ 82–107k tCO₂/yr — our 150–194 GWh generation estimate × the published CDM grid emission factor for Uzbekistan; a conservative lower bound, tracking our lower-bound generation | ~156,000 tCO₂/yr avoided | Consistent — tracks our conservative generation; the reported 270 GWh → 156k implies a grid factor just above the published CDM range for Uzbekistan |
| AOI box 545 ha; panels a subset | 100 MW footprint (~150–250 ha expected) | Plausible; confirm footprint |

Sources for reported figures (confirm each): Masdar — 100 MW Nur Navoi Solar Project (masdar.ae); pv-magazine, “Uzbekistan flicks switch on 100 MW Nur Navoi solar plant” (1 Sep 2021); IFC / World Bank Group project documents.

Biodiversity: not observable from Sentinel — needs hyperspectral or ground truth. We benchmark proxies only (habitat extent stable; NDVI no decline).